# Mother–Daughter Centre Routing — Solution Report

Prepared by: OR-Tools MDVRP solver  |  Solver: Google OR-Tools v9  |  Time budget: 600 s

## 1. Problem Statement

The task is to connect 191 daughter centres to 11 mother centres using a fleet of Tata Ace vehicles.  Each vehicle departs from its home mother centre at 06:00, delivers to one or more daughter centres (with the last delivery reaching its destination by 10:00), and then returns to the same mother centre.  The objective is to minimise total vehicle operating cost (fixed + variable).

## 2. Assumptions and Modelling Decisions

Distances are computed using the haversine great-circle formula because only GPS coordinates (latitude, longitude) are provided.  Actual road distances may be longer; the model treats haversine as a lower-bound proxy.

Vehicle speed is constant at 50 km/h for all legs (road-speed variation not modelled).

Service time at each daughter centre is zero minutes (unloading time is not specified).

Fleet size is unconstrained per mother centre — one vehicle is modelled per daughter nearest to that mother (upper bound), and the solver decides which vehicles to consolidate.

No vehicle capacity limit was specified, so cargo load is not constrained.

Every daughter centre must be assigned to exactly one mother centre and served by exactly one route.

The 10:00 AM deadline applies only to the LAST daughter stop; return to the mother can occur any time before midnight.

## 3. Solution Algorithm

The problem is modelled as a Multi-Depot Vehicle Routing Problem with Time Windows (MDVRPTW) and solved with the Google OR-Tools CP routing engine.

Step 1 — Pre-processing.  All pairwise haversine distances, travel times, and arc costs are pre-computed for the 202-node graph (11 depots + 191 customers).

Step 2 — First solution.  PARALLEL_CHEAPEST_INSERTION greedily assigns each unserved daughter to the cheapest feasible route, producing the initial incumbent.

Step 3 — Local search improvement.  GUIDED_LOCAL_SEARCH (GLS) iteratively escapes local optima by penalising arc features that appear frequently in poor solutions.  The solver runs for up to 600 seconds.

Step 4 — Lower bound.  After solving, routing.objective_lower_bound() returns the tightest lower bound the solver has proved on the global optimum.  Comparing this with the incumbent yields the optimality gap reported below.

## 4. Key Parameters

| Parameter | Value |
| --- | --- |
| Vehicle speed | 50 km/h |
| Fixed cost per vehicle | Rs 1,000 |
| Rate per km | Rs 10 |
| Rate per hour | Rs 50 |
| Effective variable cost/km | Rs 11 |
| Departure time | 06:00 |
| Last delivery deadline | 10:00 (240 min) |
| Number of mother centres | 11 |
| Number of daughter centres | 191 |
| Solver time limit | 600 s |

## 5. Results Overview

The solver found a feasible solution that serves all 191 daughter centres using 20 vehicle routes across 11 mother depots.

### 5a. Optimality Gap

| Metric | Value |
| --- | --- |
| Incumbent solution cost (Rs) | 66981.16 |
| Best lower bound (Rs) | 29182.94 |
| Absolute gap (Rs) | 37798.22 |
| Relative gap (%) | 56.4311 |
| Solver status | FEASIBLE (not proven optimal) — optimality gap = 56.4311 % |
| Solver time limit (s) | 600.0 |

Interpretation: An optimality gap > 0 % means the solver has found a feasible solution but cannot yet certify that no cheaper solution exists.  The lower bound is the tightest cost the solver has proved any solution must equal or exceed.  The gap narrows as the solver runs longer or when a tighter formulation is used.

### 5b. Cost Summary

| Metric | Value |
| --- | --- |
| Routes used | 20 |
| Total distance (km) | 4,271.02 |
| Total fixed cost (Rs) | 20,000.00 |
| Total variable cost (Rs) | 46,981.11 |
| Total cost (Rs) | 66,981.11 |

## 6. Mother Centre Assignment Summary

The table below shows how many daughter centres and how many routes are assigned to each mother centre in the optimal solution.

| Mother Centre | Daughters Assigned | Routes Used |
| --- | --- | --- |
| 57 | 10 | 1 |
| 118 | 12 | 2 |
| 124 | 22 | 2 |
| 133 | 21 | 2 |
| 134 | 15 | 1 |
| 141 | 10 | 1 |
| 142 | 14 | 1 |
| 145 | 32 | 4 |
| 171 | 21 | 2 |
| 178 | 34 | 4 |

## 7. Detailed Route Listing

Each row describes one vehicle route.  'Path' shows the sequence of centres visited (mother → daughters → mother).  'Last delivery clock' is the scheduled arrival at the final daughter stop.

| Route ID | Mother | Stops | Path | Last Delivery | Return | Distance (km) | Total Cost (Rs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M57_M57_V01 | 57 | 10 | 57 → 158 → 12 → 186 → 7 → 54 → 185 → 41 → 71 → 100 → 62 → 57 | 08:36 | 08:36 | 125.87 | 2384.55 |
| M118_M118_V02 | 118 | 7 | 118 → 22 → 6 → 8 → 14 → 81 → 195 → 60 → 118 | 09:29 | 10:07 | 202.40 | 3226.35 |
| M118_M118_V04 | 118 | 5 | 118 → 30 → 53 → 108 → 49 → 86 → 118 | 09:40 | 10:32 | 223.56 | 3459.18 |
| M124_M124_V01 | 124 | 9 | 124 → 132 → 79 → 15 → 29 → 55 → 16 → 179 → 48 → 139 → 124 | 09:24 | 10:08 | 202.61 | 3228.75 |
| M124_M124_V02 | 124 | 13 | 124 → 10 → 35 → 61 → 143 → 176 → 56 → 2 → 46 → 190 → 198 → 155 → 50 → 149 → 124 | 09:55 | 11:04 | 247.39 | 3721.24 |
| M133_M133_V01 | 133 | 10 | 133 → 106 → 136 → 163 → 1 → 153 → 123 → 121 → 32 → 28 → 164 → 133 | 09:30 | 09:53 | 189.69 | 3086.62 |
| M133_M133_V02 | 133 | 11 | 133 → 18 → 95 → 31 → 150 → 188 → 193 → 115 → 168 → 3 → 37 → 138 → 133 | 09:51 | 10:51 | 237.83 | 3616.13 |
| M134_M134_V01 | 134 | 15 | 134 → 127 → 92 → 70 → 125 → 4 → 194 → 197 → 184 → 173 → 165 → 126 → 93 → 72 → 182 → 19 → 134 | 09:55 | 10:44 | 230.80 | 3538.77 |
| M141_M141_V01 | 141 | 10 | 141 → 147 → 104 → 122 → 65 → 45 → 200 → 169 → 154 → 148 → 97 → 141 | 09:14 | 09:19 | 162.68 | 2789.52 |
| M142_M142_V01 | 142 | 14 | 142 → 66 → 98 → 80 → 51 → 25 → 74 → 13 → 159 → 64 → 202 → 157 → 99 → 102 → 88 → 142 | 09:38 | 09:38 | 177.28 | 2950.08 |
| M145_M145_V01 | 145 | 8 | 145 → 120 → 172 → 84 → 114 → 161 → 26 → 96 → 23 → 145 | 09:55 | 10:42 | 230.37 | 3534.05 |
| M145_M145_V03 | 145 | 7 | 145 → 58 → 101 → 67 → 174 → 85 → 76 → 103 → 145 | 09:56 | 12:06 | 301.95 | 4321.42 |
| M145_M145_V04 | 145 | 9 | 145 → 109 → 11 → 91 → 183 → 151 → 89 → 187 → 27 → 75 → 145 | 09:51 | 11:33 | 272.53 | 3997.87 |
| M145_M145_V05 | 145 | 8 | 145 → 94 → 140 → 52 → 36 → 39 → 83 → 47 → 73 → 145 | 08:32 | 09:21 | 162.45 | 2787.00 |
| M171_M171_V01 | 171 | 12 | 171 → 170 → 130 → 77 → 82 → 181 → 34 → 117 → 144 → 33 → 112 → 180 → 167 → 171 | 09:44 | 09:52 | 187.23 | 3059.51 |
| M171_M171_V03 | 171 | 9 | 171 → 119 → 177 → 191 → 162 → 189 → 192 → 105 → 160 → 111 → 171 | 09:37 | 10:12 | 206.02 | 3266.23 |
| M178_M178_V01 | 178 | 9 | 178 → 42 → 69 → 128 → 110 → 146 → 116 → 87 → 129 → 107 → 178 | 09:56 | 10:31 | 223.16 | 3454.73 |
| M178_M178_V02 | 178 | 10 | 178 → 135 → 196 → 152 → 40 → 78 → 199 → 21 → 131 → 156 → 90 → 178 | 09:19 | 09:55 | 190.67 | 3097.33 |
| M178_M178_V03 | 178 | 9 | 178 → 43 → 38 → 17 → 20 → 9 → 59 → 113 → 44 → 68 → 178 | 09:50 | 12:17 | 309.14 | 4400.50 |
| M178_M178_V05 | 178 | 6 | 178 → 201 → 63 → 166 → 24 → 5 → 175 → 178 | 09:21 | 09:47 | 187.39 | 3061.28 |

## 8. Complete Daughter–Mother Assignment

Every daughter centre, its assigned mother, the route it belongs to, its stop position within the route, and its scheduled arrival time.

| Daughter | Assigned Mother | Route ID | Stop # | Arrival (min) | Arrival (clock) |
| --- | --- | --- | --- | --- | --- |
| 158 | 57 | M57_M57_V01 | 1 | 8 | 06:08 |
| 12 | 57 | M57_M57_V01 | 2 | 35 | 06:35 |
| 186 | 57 | M57_M57_V01 | 3 | 56 | 06:56 |
| 7 | 57 | M57_M57_V01 | 4 | 63 | 07:03 |
| 54 | 57 | M57_M57_V01 | 5 | 78 | 07:18 |
| 185 | 57 | M57_M57_V01 | 6 | 95 | 07:35 |
| 41 | 57 | M57_M57_V01 | 7 | 141 | 08:21 |
| 71 | 57 | M57_M57_V01 | 8 | 147 | 08:27 |
| 100 | 57 | M57_M57_V01 | 9 | 153 | 08:33 |
| 62 | 57 | M57_M57_V01 | 10 | 156 | 08:36 |
| 22 | 118 | M118_M118_V02 | 1 | 6 | 06:06 |
| 6 | 118 | M118_M118_V02 | 2 | 38 | 06:38 |
| 8 | 118 | M118_M118_V02 | 3 | 72 | 07:12 |
| 14 | 118 | M118_M118_V02 | 4 | 101 | 07:41 |
| 81 | 118 | M118_M118_V02 | 5 | 142 | 08:22 |
| 195 | 118 | M118_M118_V02 | 6 | 163 | 08:43 |
| 60 | 118 | M118_M118_V02 | 7 | 209 | 09:29 |
| 30 | 118 | M118_M118_V04 | 1 | 15 | 06:15 |
| 53 | 118 | M118_M118_V04 | 2 | 55 | 06:55 |
| 108 | 118 | M118_M118_V04 | 3 | 135 | 08:15 |
| 49 | 118 | M118_M118_V04 | 4 | 185 | 09:05 |
| 86 | 118 | M118_M118_V04 | 5 | 220 | 09:40 |
| 132 | 124 | M124_M124_V01 | 1 | 0 | 06:00 |
| 79 | 124 | M124_M124_V01 | 2 | 18 | 06:18 |
| 15 | 124 | M124_M124_V01 | 3 | 71 | 07:11 |
| 29 | 124 | M124_M124_V01 | 4 | 83 | 07:23 |
| 55 | 124 | M124_M124_V01 | 5 | 106 | 07:46 |
| 16 | 124 | M124_M124_V01 | 6 | 140 | 08:20 |
| 179 | 124 | M124_M124_V01 | 7 | 158 | 08:38 |
| 48 | 124 | M124_M124_V01 | 8 | 183 | 09:03 |
| 139 | 124 | M124_M124_V01 | 9 | 204 | 09:24 |
| 10 | 124 | M124_M124_V02 | 1 | 13 | 06:13 |
| 35 | 124 | M124_M124_V02 | 2 | 22 | 06:22 |
| 61 | 124 | M124_M124_V02 | 3 | 36 | 06:36 |
| 143 | 124 | M124_M124_V02 | 4 | 42 | 06:42 |
| 176 | 124 | M124_M124_V02 | 5 | 69 | 07:09 |
| 56 | 124 | M124_M124_V02 | 6 | 91 | 07:31 |
| 2 | 124 | M124_M124_V02 | 7 | 96 | 07:36 |
| 46 | 124 | M124_M124_V02 | 8 | 123 | 08:03 |
| 190 | 124 | M124_M124_V02 | 9 | 142 | 08:22 |
| 198 | 124 | M124_M124_V02 | 10 | 171 | 08:51 |
| 155 | 124 | M124_M124_V02 | 11 | 201 | 09:21 |
| 50 | 124 | M124_M124_V02 | 12 | 207 | 09:27 |
| 149 | 124 | M124_M124_V02 | 13 | 235 | 09:55 |
| 106 | 133 | M133_M133_V01 | 1 | 17 | 06:17 |
| 136 | 133 | M133_M133_V01 | 2 | 46 | 06:46 |
| 163 | 133 | M133_M133_V01 | 3 | 61 | 07:01 |
| 1 | 133 | M133_M133_V01 | 4 | 79 | 07:19 |
| 153 | 133 | M133_M133_V01 | 5 | 115 | 07:55 |
| 123 | 133 | M133_M133_V01 | 6 | 127 | 08:07 |
| 121 | 133 | M133_M133_V01 | 7 | 164 | 08:44 |
| 32 | 133 | M133_M133_V01 | 8 | 169 | 08:49 |
| 28 | 133 | M133_M133_V01 | 9 | 183 | 09:03 |
| 164 | 133 | M133_M133_V01 | 10 | 210 | 09:30 |
| 18 | 133 | M133_M133_V02 | 1 | 0 | 06:00 |
| 95 | 133 | M133_M133_V02 | 2 | 33 | 06:33 |
| 31 | 133 | M133_M133_V02 | 3 | 84 | 07:24 |
| 150 | 133 | M133_M133_V02 | 4 | 98 | 07:38 |
| 188 | 133 | M133_M133_V02 | 5 | 119 | 07:59 |
| 193 | 133 | M133_M133_V02 | 6 | 130 | 08:10 |
| 115 | 133 | M133_M133_V02 | 7 | 151 | 08:31 |
| 168 | 133 | M133_M133_V02 | 8 | 177 | 08:57 |
| 3 | 133 | M133_M133_V02 | 9 | 192 | 09:12 |
| 37 | 133 | M133_M133_V02 | 10 | 205 | 09:25 |
| 138 | 133 | M133_M133_V02 | 11 | 231 | 09:51 |
| 127 | 134 | M134_M134_V01 | 1 | 0 | 06:00 |
| 92 | 134 | M134_M134_V01 | 2 | 4 | 06:04 |
| 70 | 134 | M134_M134_V01 | 3 | 15 | 06:15 |
| 125 | 134 | M134_M134_V01 | 4 | 23 | 06:23 |
| 4 | 134 | M134_M134_V01 | 5 | 38 | 06:38 |
| 194 | 134 | M134_M134_V01 | 6 | 50 | 06:50 |
| 197 | 134 | M134_M134_V01 | 7 | 61 | 07:01 |
| 184 | 134 | M134_M134_V01 | 8 | 89 | 07:29 |
| 173 | 134 | M134_M134_V01 | 9 | 114 | 07:54 |
| 165 | 134 | M134_M134_V01 | 10 | 149 | 08:29 |
| 126 | 134 | M134_M134_V01 | 11 | 164 | 08:44 |
| 93 | 134 | M134_M134_V01 | 12 | 182 | 09:02 |
| 72 | 134 | M134_M134_V01 | 13 | 200 | 09:20 |
| 182 | 134 | M134_M134_V01 | 14 | 221 | 09:41 |
| 19 | 134 | M134_M134_V01 | 15 | 235 | 09:55 |
| 147 | 141 | M141_M141_V01 | 1 | 0 | 06:00 |
| 104 | 141 | M141_M141_V01 | 2 | 6 | 06:06 |
| 122 | 141 | M141_M141_V01 | 3 | 21 | 06:21 |
| 65 | 141 | M141_M141_V01 | 4 | 45 | 06:45 |
| 45 | 141 | M141_M141_V01 | 5 | 70 | 07:10 |
| 200 | 141 | M141_M141_V01 | 6 | 100 | 07:40 |
| 169 | 141 | M141_M141_V01 | 7 | 128 | 08:08 |
| 154 | 141 | M141_M141_V01 | 8 | 158 | 08:38 |
| 148 | 141 | M141_M141_V01 | 9 | 187 | 09:07 |
| 97 | 141 | M141_M141_V01 | 10 | 194 | 09:14 |
| 66 | 142 | M142_M142_V01 | 1 | 6 | 06:06 |
| 98 | 142 | M142_M142_V01 | 2 | 20 | 06:20 |
| 80 | 142 | M142_M142_V01 | 3 | 44 | 06:44 |
| 51 | 142 | M142_M142_V01 | 4 | 63 | 07:03 |
| 25 | 142 | M142_M142_V01 | 5 | 79 | 07:19 |
| 74 | 142 | M142_M142_V01 | 6 | 94 | 07:34 |
| 13 | 142 | M142_M142_V01 | 7 | 113 | 07:53 |
| 159 | 142 | M142_M142_V01 | 8 | 128 | 08:08 |
| 64 | 142 | M142_M142_V01 | 9 | 184 | 09:04 |
| 202 | 142 | M142_M142_V01 | 10 | 193 | 09:13 |
| 157 | 142 | M142_M142_V01 | 11 | 201 | 09:21 |
| 99 | 142 | M142_M142_V01 | 12 | 206 | 09:26 |
| 102 | 142 | M142_M142_V01 | 13 | 211 | 09:31 |
| 88 | 142 | M142_M142_V01 | 14 | 218 | 09:38 |
| 120 | 145 | M145_M145_V01 | 1 | 4 | 06:04 |
| 172 | 145 | M145_M145_V01 | 2 | 18 | 06:18 |
| 84 | 145 | M145_M145_V01 | 3 | 127 | 08:07 |
| 114 | 145 | M145_M145_V01 | 4 | 144 | 08:24 |
| 161 | 145 | M145_M145_V01 | 5 | 186 | 09:06 |
| 26 | 145 | M145_M145_V01 | 6 | 212 | 09:32 |
| 96 | 145 | M145_M145_V01 | 7 | 213 | 09:33 |
| 23 | 145 | M145_M145_V01 | 8 | 235 | 09:55 |
| 58 | 145 | M145_M145_V03 | 1 | 36 | 06:36 |
| 101 | 145 | M145_M145_V03 | 2 | 72 | 07:12 |
| 67 | 145 | M145_M145_V03 | 3 | 114 | 07:54 |
| 174 | 145 | M145_M145_V03 | 4 | 136 | 08:16 |
| 85 | 145 | M145_M145_V03 | 5 | 184 | 09:04 |
| 76 | 145 | M145_M145_V03 | 6 | 203 | 09:23 |
| 103 | 145 | M145_M145_V03 | 7 | 236 | 09:56 |
| 109 | 145 | M145_M145_V04 | 1 | 39 | 06:39 |
| 11 | 145 | M145_M145_V04 | 2 | 51 | 06:51 |
| 91 | 145 | M145_M145_V04 | 3 | 95 | 07:35 |
| 183 | 145 | M145_M145_V04 | 4 | 97 | 07:37 |
| 151 | 145 | M145_M145_V04 | 5 | 147 | 08:27 |
| 89 | 145 | M145_M145_V04 | 6 | 163 | 08:43 |
| 187 | 145 | M145_M145_V04 | 7 | 188 | 09:08 |
| 27 | 145 | M145_M145_V04 | 8 | 189 | 09:09 |
| 75 | 145 | M145_M145_V04 | 9 | 231 | 09:51 |
| 94 | 145 | M145_M145_V05 | 1 | 7 | 06:07 |
| 140 | 145 | M145_M145_V05 | 2 | 38 | 06:38 |
| 52 | 145 | M145_M145_V05 | 3 | 46 | 06:46 |
| 36 | 145 | M145_M145_V05 | 4 | 68 | 07:08 |
| 39 | 145 | M145_M145_V05 | 5 | 88 | 07:28 |
| 83 | 145 | M145_M145_V05 | 6 | 94 | 07:34 |
| 47 | 145 | M145_M145_V05 | 7 | 110 | 07:50 |
| 73 | 145 | M145_M145_V05 | 8 | 152 | 08:32 |
| 170 | 171 | M171_M171_V01 | 1 | 0 | 06:00 |
| 130 | 171 | M171_M171_V01 | 2 | 37 | 06:37 |
| 77 | 171 | M171_M171_V01 | 3 | 66 | 07:06 |
| 82 | 171 | M171_M171_V01 | 4 | 69 | 07:09 |
| 181 | 171 | M171_M171_V01 | 5 | 70 | 07:10 |
| 34 | 171 | M171_M171_V01 | 6 | 99 | 07:39 |
| 117 | 171 | M171_M171_V01 | 7 | 112 | 07:52 |
| 144 | 171 | M171_M171_V01 | 8 | 128 | 08:08 |
| 33 | 171 | M171_M171_V01 | 9 | 151 | 08:31 |
| 112 | 171 | M171_M171_V01 | 10 | 163 | 08:43 |
| 180 | 171 | M171_M171_V01 | 11 | 194 | 09:14 |
| 167 | 171 | M171_M171_V01 | 12 | 224 | 09:44 |
| 119 | 171 | M171_M171_V03 | 1 | 23 | 06:23 |
| 177 | 171 | M171_M171_V03 | 2 | 43 | 06:43 |
| 191 | 171 | M171_M171_V03 | 3 | 76 | 07:16 |
| 162 | 171 | M171_M171_V03 | 4 | 89 | 07:29 |
| 189 | 171 | M171_M171_V03 | 5 | 117 | 07:57 |
| 192 | 171 | M171_M171_V03 | 6 | 143 | 08:23 |
| 105 | 171 | M171_M171_V03 | 7 | 174 | 08:54 |
| 160 | 171 | M171_M171_V03 | 8 | 194 | 09:14 |
| 111 | 171 | M171_M171_V03 | 9 | 217 | 09:37 |
| 42 | 178 | M178_M178_V01 | 1 | 16 | 06:16 |
| 69 | 178 | M178_M178_V01 | 2 | 52 | 06:52 |
| 128 | 178 | M178_M178_V01 | 3 | 54 | 06:54 |
| 110 | 178 | M178_M178_V01 | 4 | 57 | 06:57 |
| 146 | 178 | M178_M178_V01 | 5 | 111 | 07:51 |
| 116 | 178 | M178_M178_V01 | 6 | 138 | 08:18 |
| 87 | 178 | M178_M178_V01 | 7 | 166 | 08:46 |
| 129 | 178 | M178_M178_V01 | 8 | 202 | 09:22 |
| 107 | 178 | M178_M178_V01 | 9 | 236 | 09:56 |
| 135 | 178 | M178_M178_V02 | 1 | 1 | 06:01 |
| 196 | 178 | M178_M178_V02 | 2 | 22 | 06:22 |
| 152 | 178 | M178_M178_V02 | 3 | 59 | 06:59 |
| 40 | 178 | M178_M178_V02 | 4 | 82 | 07:22 |
| 78 | 178 | M178_M178_V02 | 5 | 86 | 07:26 |
| 199 | 178 | M178_M178_V02 | 6 | 110 | 07:50 |
| 21 | 178 | M178_M178_V02 | 7 | 126 | 08:06 |
| 131 | 178 | M178_M178_V02 | 8 | 164 | 08:44 |
| 156 | 178 | M178_M178_V02 | 9 | 177 | 08:57 |
| 90 | 178 | M178_M178_V02 | 10 | 199 | 09:19 |
| 43 | 178 | M178_M178_V03 | 1 | 0 | 06:00 |
| 38 | 178 | M178_M178_V03 | 2 | 29 | 06:29 |
| 17 | 178 | M178_M178_V03 | 3 | 53 | 06:53 |
| 20 | 178 | M178_M178_V03 | 4 | 92 | 07:32 |
| 9 | 178 | M178_M178_V03 | 5 | 128 | 08:08 |
| 59 | 178 | M178_M178_V03 | 6 | 156 | 08:36 |
| 113 | 178 | M178_M178_V03 | 7 | 180 | 09:00 |
| 44 | 178 | M178_M178_V03 | 8 | 205 | 09:25 |
| 68 | 178 | M178_M178_V03 | 9 | 230 | 09:50 |
| 201 | 178 | M178_M178_V05 | 1 | 40 | 06:40 |
| 63 | 178 | M178_M178_V05 | 2 | 52 | 06:52 |
| 166 | 178 | M178_M178_V05 | 3 | 68 | 07:08 |
| 24 | 178 | M178_M178_V05 | 4 | 129 | 08:09 |
| 5 | 178 | M178_M178_V05 | 5 | 148 | 08:28 |
| 175 | 178 | M178_M178_V05 | 6 | 201 | 09:21 |

## 9. Route Map

The figure below shows all mother centres (★) and daughter centres (●) on a latitude/longitude map.  Lines represent vehicle routes; colour identifies the home mother centre.